Colour code:

Known.

Not known. (Please see the bottom of page 2 and the top of page 3.)

Equations concerning the medium

Material discontinuity:

nt: Unit vector in the direction of the refracted ray.

nt=ηni+γns

η: Ratio of refractive indices; Units: 1.

η=n1n2

n1: Complex refractive index of the medium surrounding the unit lenses.

n2: Complex refractive index of the unit lens material (the P-doped Si).

ni: Unit vector in the direction of the incident ray.

γ: (Just a help variable.)

γ=-ηcosθi+cosθt

θi: Angle of incidence. Units: rad.

θt: Angle of refraction. Units: rad.

θt=asinηsinθi

ns: Unit vector normal to the material discontinuity.

tparallel: Transmission coefficient, parallel (p) polarised.

tparallel=2n1cosθin2cosθi+n1cosθt

tperpendicular: Transmission coefficient, perpendicular (s) polarised.

tperpendicular=2n1cosθin1cosθi+n2cosθt

ku, t=1ru, t: uth principal wavefront curvature of refracted rays defined for direction eu. Units: m-1.

ru, t: uth principal radius of curvature of the refracted rays. Units: m.

ku, t=ηku, i+γku, s

ku, i: uth principal wavefront curvature of incident rays. Units: m-1.

u, i: Vector tangent to incident wavefront.

ku, s: uth principal curvature of the surface of the unit lenses. Units: m-1.

kuv, t: Off-diagonal element uv in the wavefront curvature tensor of the refracted rays. Units: m-1.

kuv, t=ηcosθicosθtkuv, i+γcosθtkuv, s

kuv, i: Off-diagonal element uv in the wavefront curvature tensor of the incident rays. Units: m-1.

kuv, s: Off-diagonal element uv in the curvature tensor of the surface of the unit lenses. Units: m-1.

kv, t=1rv, t: vth principal wavefront curvature of refracted rays defined for direction ev. Units: m-1.

rv, t: vth principal radius of curvature of the refracted rays. Units: m.

kv, t=ηcos2θicos2θtkv, i+γcos2θtkv, s

kv, i: vth principal wavefront curvature of incident rays. Units: m-1.

kv, s: vth principal curvature of the surface of the unit lenses. Units: m-1.

Equations concerning the rays

Release rays from grid:

k: Wave vector of the rays; Units: rad m-1.

k=k0L0L0

k0=2πfc: Wave number of the rays in vacuum. Units: rad m-1.

f: frequency of the rays in vacuum. Units: Hz.

c: Speed of light in vacuum, 299,792,458 m s-1.

L0: Initial ray direction vector; {0, 1, 0}.

Ray properties (Monochromatic, 23.0 keV):

Initial conditions:

- Unpolarised plane wave.

Geometric optics:

dqdt=∂ω∂k  Units: ms=rad s-1rad m-1

t: time since the rays were released. Units: s.

ω: Angular frequency of the rays; Units: rad s-1.

q: Ray position vector; Units: m.

dkdt=-∂ω∂q

Ray properties:

“In 3D models [such as the one used in this work], each ray is treated as a wavefront for which two principal radii of curvature, r1 and r2, are defined” (COMSOL Inc., 2018).

dsi, 0ds=-2kκsi, 0	i∈0, 1, 2, 3 ; Units: W m-2m=1rad m-11W m-2

si, 0: Initial Stokes parameter i; Units: W m-2.

κ: the imaginary part of the refractive index.

si=r1, 0r2, 0r1r2si, 0

si: Stokes parameter i; Units: W m-2.

r1, 0: First initial principal radius of curvature of the rays; Units: m.

r2, 0: Second initial principal radius of curvature of the rays; Units: m.

dr1ds=-1, within the unit lenses.

r1 and r2: First and second principal radii of curvature of the rays, “which represent the maximum and minimum radii of curvature of the intersection of the wavefront with an arbitrary plane” (COMSOL Inc., 2018); Units: m.

s: Distance that the rays have travelled; Units: m?  Ray trajectory?

dr2ds=-1, within the unit lenses.

Deposited ray power:

∂Qsrc∂t=-j=1Nt∂Qj∂tδr-qj ; Units: W m-3s=W m-3sm-m

In this expression every ray is treated as an infinitesimally small point source.

Qsrc: Heat power deposited into the unit lenses due to the rays; Units: W m-3.

Qj: The amount of power lost by ray j as it transfers power into the unit lenses;Units: W m-3.

Nt: Total number of rays.

j: Summation index for the rays.

r: Position vectors of points that constitute the unit lenses; Units: m.

qj: Position vector of ray j; Units: m.

δ: Dirac delta function.

Ray termination:

If Q<Qth or I<Ith, disappear

Q: Power of any ray that is being simulated. Units: W.

Qth: Threshold/minimum power of the rays for them to be simulated. Units: W.

I: Intensity of any ray that is being simulated. Units: W m-2.

Ith: Threshold/minimum intensity of the rays for them to be simulated. Units: W m-2.

Equations concerning heat transfer

Ray heat source (Multiphysics):

(Same equation as that for Deposited ray power.)

Heat transfer in solids:

ρCpu⋅∇T+∇⋅q=Q+Qted ; Units: kg m-3J kg-1 K-1m s-1K ?+?W m-2=W m-3  m-2Js-1?+?W m-2=W m-3  m-2W?+?W m-2=W m-3  W m-2?=W m-3  ?=m-1

ρCpu⋅∇T-k∇2T=Q+Qted

ρ: Mass density of the material (the P-doped Si). Units: kg m-3.

Cp: Heat capacity of the material (the P-doped Si) at constant pressure. Units: J kg-1 K-1.

u: Fluid Velocity field of the unit lenses; Units: m s-1. The x, y and z components of u all equal 0 m s-1 because the unit lenses were in solid phase.

T: Temperature of the material (the P-doped Si). Units: K.

∇: Gradient vector of the surface normal; Units: m-1.

q: Conductive heat flux vector; Units: W m-2.

q=-k∇T (The unit lenses are a continuous medium)

k: Thermal conductivity of the material (the P-doped Si). Units: W m-1 K-1.

∇T: Temperature gradient in the direction of the surface normal vector n. Units: K m-1.

Q: Heat source for the Heat Equation; Units: W m-3.

Qted: Thermoelastic damping in the unit lenses. Units: W m-3.

Thermal insulation of the right-side faces of the unit lenses:

-n⋅q=0

n: The vector normal to each face.

Surface-to-ambient radiation:

-n⋅q=εσTamb4-T4

-n⋅q: Radiative heat flux; Units: W m-2.

ε: Surface emissivity of the material (the P-doped Si).

σ: Stefan-Boltzmann constant.

Tamb=293.15 K: Ambient temperature.

References

COMSOL Inc. (2018). Ray Optics Module User's Guide, 5.4 ed. Internet: COMSOL Inc.